NAME : BISMA FATIMA

CLASS : CS A

ROLL NO: F24605036

COURSE : COMPUTER NETWORK

COURSE CODE : CS-271

LECTURER : SIR NAVEED YOUSAF

LAB  10

Configuration of DHCP in VLANs

Warm up Task [30 Minutes]

- Answer each question in your own words (3 to 4 lines)

1. What is VAN?

- VAN (Value-Added Network) is a private network provider that offers specialized services beyond basic data transmission.

- It acts as an intermediary between businesses, often used for secure Electronic Data Interchange (EDI), providing services like encryption, message routing, and auditing.

- Think of it as a managed communication service that adds reliability and security compared to using the public internet.

2. How are VLAN identified?

- VLANs are identified by a VLAN ID, which is a numerical value assigned to each VLAN.

- The VLAN ID is typically a 12-bit field in the Ethernet frame, allowing up to 4096 VLANs (0–4095), though IDs 0 and 4095 are reserved.

- Network switches use these IDs to segregate traffic logically, even if devices share the same physical infrastructure.

3. What is VLAN tagging?

- VLAN tagging is the process of inserting a VLAN ID into Ethernet frames so that devices and switches know which VLAN the traffic belongs to.

- This is done using the IEEE 802.1Q standard, which adds a 4-byte tag to the Ethernet frame header.

- Tagging is essential when traffic from multiple VLANs travels across a single physical link (called a trunk port).

4. How does a VLAN differ from a physical LAN?

- Physical LAN: Devices are grouped based on physical connections (same switch, same cabling).

- VLAN (Virtual LAN): Devices are grouped logically, regardless of physical location, using VLAN IDs.

- Key differences:

  - VLANs provide segmentation without requiring separate hardware.

  - VLANs improve security and traffic management by isolating broadcast domains.

  - Physical LANs are limited by hardware layout, while VLANs are flexible and scalable.

5. What is the primary purpose of DHCP?

- The primary purpose of DHCP (Dynamic Host Configuration Protocol) is to automatically assign IP configuration parameters to devices on a network.

- This eliminates the need for manual configuration, reduces errors, and ensures efficient IP address management.

6. Name three key configuration parameters provided by DHCP.

DHCP typically provides:

- IP Address – The unique identifier for the device on the network.

- Subnet Mask – Defines the network and host portions of the IP address.

- Default Gateway – The router address used to reach other networks.

Additional parameters can include DNS server addresses, lease time, and more.

TASK 1

By using Drag and drop draw topology diagram as Shown below and attach screenshot of each step.

Figure 1. Packet Tracer Topology Diagram

Configuration DHCP for Inter-Vlan by Using Router

Step1: Configure switch for Vlan

Step2: After creating Vlan connect a router to switch through fast Ethernet interface

Switch configuration for Vlan

- en

- Config t

- Int fa0/7(switch interface connected with router)

- Switchport mode trunk

- Exit

- Exit

Step3: Router configuration for Vlan

- en

- Show ip interface brief

- Config t

- Int fa0/0(router interface connected with switch)

- No shutdown

- Exit

- Exit

- Show ip interface brief

Creation of sub Interface and route

- en

- Show ip interface brief

- Config t

- Int fa0/0.1     (router interface connected with switch)

- Encapsulation dot1Q 2

- Ip address 192.168.10.100 255.255.255.0

- Exit

- Int fa0/0.2     (router interface connected with switch)

- Encapsulation dot1Q 5

- Ip address 192.168.11.100 255.255.255.0

- Exit

- Exit

Step4: DHCP configuration in router

- en

- Config t

- Ip dhcp pool 1

- Network 192.168.10.0 255.255.255.0

- Default-router 192.168.10.100

- Exit

- Ip dhcp excluded-address 192.168.10.100 192.168.10.105

- Exit

- Repeat step 4 for other networks

Realtime :

TASK 2

Draw given topology diagrams and attach screen shots of Pcs communication

- Highlight different sections

- Divide given topology in four different networks

- Ping different VLANs laptops.

- Send packets (Real time).

- Send packets(Simulation).

- Take screen shots of every step

Note*

All screen shots and topology labeling must be clearly visible.

Figure 2. Task 2 topology diagram

Step1: Configure switch for Vlan

Step2: After creating Vlan connect a router to switch through fast Ethernet interface

Switch configuration for Vlan

Step3: Router configuration for Vlan

Creation of sub Interface and route

Step4: DHCP configuration in router

Show ip interface brief:

Pinging:

Real time:

VLAN brief: